Deadlaus Mod is a toolhead-focused upgrade for the QIDI Q1 Pro, aimed at reliable high-temperature, enclosed, technical printing.

It adds a Klicky-style detachable probe, moves the hotend to the Volcano nozzle ecosystem (incl. CHT inserts), upgrades hotend cooling and part cooling, and adds an extra rear-cover fan to reduce heat soak in a hot chamber.

Fully reversible to stock (no permanent mods required — you can return to the original toolhead setup).

#### Highlights

- Klicky detachable probe for accurate Z and flexible build surfaces

- Volcano nozzle system (supports CHT inserts) for higher flow and stronger parts

- Upgraded hotend cooling fan to 3010

- Upgraded part cooling duct

- Added rear-cover fan to prevent toolhead overheating in a hot chamber

- Designed for enclosed engineering materials workflow

- Detachable front cover thanks to a pogo-pin connection for the part cooling fan

- Rear electronics exhaust fan relocated to the back wall to pull hot air away from the electronics bay and improve temperature stability in enclosed setups

- Deterministic XY motion tuning (StealthChop disabled) for more repeatable high-speed moves and cleaner motion behavior

- Reversible mod: can be restored back to stock configuration

#### Pros

- Proven hot-chamber readiness: stable printing at 80°C chamber (tested), likely higher depending on your setup

- Works on basically any build surface (including glass) thanks to Klicky probing and consistent Z referencing

- Stronger parts / higher throughput using Volcano nozzles + CHT inserts (more flow headroom, better layer bonding at speed)

- Toolhead stays cooler in an enclosed printer: rear-cover fan reduces heat soak and helps protect electronics

- More repeatable first layer after maintenance: detachable probe reduces “drift” from fixed sensors

- More repeatable motion at high accel: XY drivers run in a consistent regime (no StealthChop), improving torque consistency for aggressive CoreXY profiles (at the cost of extra motor noise)

#### Cons

- Harder extruder access: servicing the extruder or clearing clogs takes longer due to the toolhead layout and added parts

- Slightly reduced printable area near the Klicky dock zone (rear notch). Keep models out of that region to avoid collisions

- More noise (by design): X/Y run in a consistent non-StealthChop regime for repeatability at high accel + extra fans add airflow noise

  - You can reduce noise by lowering accel/jerk, reducing fan speeds, or enabling StealthChop below a speed threshold (if you accept less deterministic motion)

- Reduced maximum Z height: using a ~5 mm glass build plate and longer Volcano nozzle setups can slightly reduce available Z travel

- No automatic nozzle wipe/cleaning: use a manual wipe/brush or a purge line in your start G-code

- Part cooling asymmetry: even though the duct is designed for balanced flow, one side may cool slightly better due to outlet geometry

### Disclaimer

This is not a beginner mod. You should be comfortable with disassembly, wiring, tuning, and Klipper configuration. Expect to recalibrate after installation (Z offset, probing workflow, temperatures/flow, PA, cooling, etc.).

Use at your own risk: everything is provided as-is. The author assumes no responsibility for any damage, malfunction, injury, or loss.

### BOM (Bill of Materials)

#### Main fasteners

- Self-tapping screws, hex socket M3×8 (outside duct → front cover)

- Self-tapping screw, hex socket M2.6×6 (inside duct → front cover)

- Countersunk screws M3×16 (5015 blower fan → front cover)

- Screws, hex socket M3×16 (rear/chamber fan → printer back wall)

- Screws, hex socket M3×15 (pogo pin mount → toolhead)

- Screws, hex socket M3×8 (Klicky holder → printhead)

- Flat head screws, hex socket M2.5×6 (extruder fan duct → radiator/heatsink)

- Flat head screws, hex socket M2.5×12 (fan → rear cover)

- Flat head screws, hex socket M2.5×16 (3010 extruder fan → extruder fan duct)

- Heat-set inserts M3 (blower fan, rear/chamber fan mounts, etc.)

- Heat-set inserts M2.5 (adapter plate fan mounts)

- Nylon zip tie 2.5 mm (printhead cable management)

- Disc Magnets 5x2 mm (front cover)

#### Fans

- Blower fan 5015 24V (stock fan not compatible)

- Extruder / hotend fan 3010 24V

- Adapter plate fan 3010 5V (note the voltage)

#### Electrical / contacts

- Pogo Pin Connector Pair SHIDWJ 2.54 mm

- Heatshrink tubing

#### Klicky standard hardware

- Klicky probe Kit (D2hw recommended)

#### Hardware

- E3D V6 Volcano CHT hardened nozzle (example)

- Glass/mirror 250x250 mm (see on your local store)

- Glue Stick PVP

- Heat insulation cotton (high-temp rated / fire-retardant)

- Acoustic foam (fire-retardant / high-temp rated)

- Metal Binder Clips

#### Tools

- Soldering iron + solder + flux (pogo pins)

- Heat-set insert tool (or soldering iron tip for inserts)

- JST-XH 2.54 crimp tool + connectors (example)

- Standard hex keys

- Precision screwdriver set (PH1 for Klicky screws, T6H for M2.5 flat screws; Torx gives better grip than hex)

### Printing recommendations

I recommend print all parts from PC-CF material. It has one of the best heat resistance and little creeping. Use minimum part fan cooling possible to achieve better layer adhesion. All models optimized for 0.2 layer height.

### Final tuning

After printing and installing all parts, replace your printer.cfg with the new one (see the latest version on GitHub).

Because every printer is a bit different, verify Klicky docking/undocking before doing full homing:

- Manually move the toolhead by hand (motors disabled) and confirm the Klicky can dock and undock without binding.

- Run G28 X Y.

- Using the coordinates in printer.cfg, move to the Approach point first, then creep toward the Dock point in small steps. Dock/undock a few times to confirm it’s reliable.

- If docking/undocking is stable, run a full G28.

After that, recalibrate as needed:

- PID_CALIBRATE (hotend, chamber and bed) — do this with the chamber and bed heated to your typical printing temps

- Belt tension check/adjust

- PROBE_CALIBRATE / Z offset

- SHAPER_CALIBRATE

- SCREWS_TILT_CALCULATE

- Bed mesh calibration

- Optional: X/Y rotation distance (steps/mm)

I recommend doing all calibration with the chamber heated to your typical printing temperature (including belt tension).

Note: Avoid relying on SAVE_CONFIG if it is unstable on your setup. Instead, copy the final calibrated values into the correct sections of the config manually.

More detailed step-by-step instructions are available on Boosty and Patreon.

### Additional required projects (find on Printables*)

- Qidi Q1 Pro Rear TPU Seal

- Qidi Q1 Pro Heat Loss Plug

- Qidi Q1 Pro Riser

- Qidi Q1 Pro Belt Tension block

*can't put links on that

Tags:

qidi, qidi-q1-pro, q1-pro, toolhead, mod, upgrade, klipper, klicky, detachable-probe, volcano, cht, high-temperature, enclosed, pc-cf, abs, asa, nylon, pa-cf, cooling-duct, fan-duct, pogo-pins, corexy